Testing Document

CODING:

- First you have to change the SSID and PASSWORD of node.

- Open the “IOT_Sensor_Interfacing_node1.ino” file.

In ssid you have to place your user name of Wi-Fi.

And in password, the respective password of Wi-Fi.

- Add the libraries

- PubSubClient

- pubsubclient-master

- PubSubClientTools

- For code in ESP8266 , you have to add the ESP8266 board in Arduino through board manager.

- Open the file in Arduino and click on preferences

- And copy this link “http://arduino.esp8266.com/stable/package_esp8266com_index.json “ in below mention part. And then press ok.

- Now go to Tools>Board>Board manager

- Now type the ESP8266 In search window and click on install button.

- Now the ESP Board has been added to your Arduino.

- Now burn the code by selecting the board shown in below mention fig.

HARDWARE:

- Verify the component as mention in BOM , right component should be place at right position.

- Now check the power supply of PCB , by placing jumper at J10

Check the respective voltages here, if not coming then test the pcb and verify the components

- Now in Arduino Uno genuine board, upload the blink code and check is it working condition?

- Interface the LCD with Arduino Uno having pin numbers:

- ACTUAL SYSTEM

- RS:   D10

- EN:  D11

- D4:  D12

- D5:  D13

- D6:  D6

- D7:  D5

- CONTROL SYSTEM

- RS:   D13

- EN:  D12

- D4:  D11

- D5:  D13

- D6:  D6

- D7:  D5

- Then check the buzzer interfacing,

- ACTUAL SYSTEM

- buzzer = A2

- CONTROL SYSTEM

- buzzer = D7

- Check the sensors i.e. Magnetic Sensors and IR sensors, is these working or not?

- Now make all the connection as per mentioned in connection diagram doc.

- Now for ESP8266 there is small modification for PCB Model no.1197

Cut the these two pins of ESP8266

- Now place the esp8266 and Arduino Uno at right place and burn the actual code.

- Don’t place jumper j21 between A3 and VD

CLOUD MQTT (create new account for every order)

- Go to  https://www.cloudmqtt.com/

- Create a new gmail account.

- Create a new account on cloud mqtt with newly create gmail.

- Login into account

- At the right side of page there is button to create new instance , click on it.

- Now make a instance

- Now click on instance e.g. here it is IOT_Test

- From details you get the information , now update this information into the code “mqtt_node”

For example:

const char *mqtt_server = "m23.cloudmqtt.com";

const int mqtt_port = 15614;

const char *mqtt_user = "fkfpjzsx";

const char *mqtt_pass = "JdulLTIH_sGW";